# SM750 WinCE Driver Specification

- Display Outputs

WinCE driver supports dual displays with simultaneous content (SIMUL mode) and different contents in one of the following output configurations:

- Digital TFT (fixed resolution) + CRT

- Digital TFT (fixed resolution) + Digital TFT (fixed resolution)

- Dual CRT

The tables below summarize the display layers for each output configuration.

- Digital TFT (fixed resolution) + CRT

|  | TFT | CRT |
| --- | --- | --- |
| Graphic layer | 18/24/36 bit digital TFT with a fixed resolution equal to panel size | Analog DAC |
| Cursor layer | Hardware | Hardware |
| Video | Overlay | 2D Color Space Conversion |
| Alpha layers | Support | N/A |

- Digital TFT (fixed resolution) + Digital TFT (fixed resolution)

|  | TFT 1 | TFT 2 |
| --- | --- | --- |
| Graphic layer | 18 bit TFT with a fixed resolution equal to panel size | 18 bit TFT with a fixed resolution equal to panel size. Note that it can use a size different from TFT 1. |
| Cursor layer | Hardware | Hardware |
| Video | Overlay | 2D Color Space Conversion |
| Alpha layers | Support | N/A |

- Dual CRT

|  | CRT 1 | CRT 2 |
| --- | --- | --- |
| Graphic layer | Analog DAC | Analog DAC |
| Cursor | Hardware | Hardware |
| Video | Overlay | 2D Color Space Conversion |
| Alpha layers | Support | N/A |

1.1 Display Output Behavior of Driver during Boot-Up

CE driver uses system REG file to set the following boot behavior:

- Resolution and color depth

- CRT or TFT display

- Panel size

- Panel type (18/24/36 bit TFT)

- Dual display or single display

- For dual display: SIMUL mode or different contents.

If there is no start up configuration, CE starts in

- 800x600x16BPP

- TFT (24 bit) + CRT in SIMUL mode

- Driver Features

| Features | Remark | Schedule / Version |
| --- | --- | --- |
| CE versions | CE 5.0 CE 6.0 | 8/28/2009 |
| 4:3 resolutions: | 640x480 800x600 1024x768 1280x1024 1600x1200 | 8/28/2009 |
| Wide screen resolutions: | 1024x600 1280x720 1360x768 1440x960 1920x1080 | 8/28/2009 |
| Color depth: 8, 16, 32 BPP | 32 BPP is not available in 1600x1200 and 1920x1080 | 8/28/2009 |
| Refresh Rates for CRT output: 60, 75, 85Hz | 85Hz is not available in 1600x1200 and 1902x1080 Wide screen resolutions only have 60Hz | 8/28/2009 |
| Panel type | 18/28 bit TFT 36 bit double pixel TFT | 8/28/2009 |
| 2D acceleration | Screen BLT HOST BLT Monochrome Font Line Draw Rectangle Fill | 8/28/2009 |
| Hardware Video | Driver will choose overlay or CSC automatically according to display mode | 8/28/2009 |
| CE Direct Draw |  | 8/28/2009 |
| Power Management | ACPI | 8/28/2009 |
| Dual Display | Support SIMUL and different contents for primary and secondary outputs from a single chip | 8/28/2009 |
| CE 6.0 Logo | Pass CETK 6.0 Test Kit | Mid Oct 2009 |
| Multiple Adapters | Two or more SM750 chips in one system. | To be scheduled |
| Rotation Modes | - Windows desktop rotates in 90, 270 and 180 degrees. - Dual Display is not supported. | To be scheduled |
| OS Veriosn | CE 4.2 | To be scheduled when request. |

Appendix:

- parameter list for the CE boot up configurations in REG file.

Boot up configuration settings include following parts:

- Resolution and color depth

- CRT or TFT display

- Panel size

- Panel type (18/24/36 bit TFT)

- Dual display or single display

- For dual display: SIMUL mode or different contents.

Following is detailed definitions if REG file.

Registry Settings

-----------------

Registry Settings determine the mode sets by the driver when it first loads up

(See Setup section on how to specify them).

* The default registry format is as follows:

[HKEY_LOCAL_MACHINE\Drivers\Display\SMIVGX]

; For Screen Size, Panel Size, and Bpp

"CxPanel"		=	dword:{any value in hex, as supported by driver}

"CyPanel"		=	dword:{any value in hex, as supported by driver}

"CxScreen"		=	dword:{any value in hex, as supported by driver}

"CyScreen"		=	dword:{any value in hex, as supported by driver}

"Bpp"	  	=	dword:{8, 10, 20}

"RefreshRate"	=	dword:{any value in hex, as supported by driver}

"TestClock"		=	dword:{any value in hex, as installed on the VGX board}

"DisplayType"	=	dword:{1 = panel, 2 = CRT, 3 = panel + CRT, 4 = panel1 + panel2 }

"DisplayControl "	=	dword:{0 = panel channel, 1 = CRT channel, 2 = VGA channel }

"SaveRestore"	=	dword:{0, 1}

"SaveSurface"	=	dword:{0, 1}

; For Host-based driver

"FB_BASE" 	= dword:{base physical address in hex}

"FG_BASE" 	= dword:{register physical address in hex}

; For SMI Special Modes and Panel Types

"PanelType" = dword:{as specified below}

; For special panel settings by modifying hardware register directly

; See definition below for detail explanation.

"PanelSettings" = hex:xx0,xx0,xx0,xx0, xx1,xx1,xx1,xx1,....

; For Rotation Feature

"Angle"		=	dword:{0,5A,B4,10E}

; For Multimon support

[HKEY_LOCAL_MACHINE\SYSTEM\GDI\MONITORS]

"TOTAL MONITORS" = dword:{number of monitors}

; NOTE: Bpp option has to be the same across all monitors (WinCE limitation)

[HKEY_LOCAL_MACHINE\Drivers\Display\SMIVGX\MONITOR0]

; Specify Panel settings and refresh rate for first monitor, as necessary

[HKEY_LOCAL_MACHINE\Drivers\Display\SMIVGX\MONITOR1]

; Specify Panel settings and refresh rate for second monitor, as necessary

[HKEY_LOCAL_MACHINE\Drivers\Display\SMIVGX\MONITOR2]

; Specify Panel settings and refresh rate for third monitor, as necessary

[HKEY_LOCAL_MACHINE\Drivers\Display\SMIVGX\MONITOR3]

; Specify Panel settings and refresh rate for fourth monitor, as necessary

* The definitions are:

- CxPanel, CyPanel: Panel width and height.

If not specified, CxPanel and CyPanel will be defaulted to 800x600

- CxScreen, CyScreen: Screen width and height (Display mode resolution).

If not specified, CxScreen and CyScreen will be the same as on

CxPanel and CyPanel.

NOTE1: currently CxScreen & CyScreen not equals CxPanel & CyPanel is not supported.

- Bpp: Number of bits of colors for one pixel.

If not specified, Bpp is defaulted to 16-bpp mode.

- RefreshRate: Refresh rate / vertical frequency of monitors/panels.

If not specified, RefreshRate is defaulted to 60 Hz.

- DisplayType:

1 - Enable panel output only

2 - Enable CRT output only

3 - Enable both panel & CRT outputs

4 - Enable both panel & CRT outputs

- DisplayControl:

0 - Use Panel output channel

1 - Use CRT   output channel

2 – use VGA   output channel

- SaveRestore: 0 - The driver will not save/restore VGX registers and memory

to/from system memory

1 - Otherwise

- SaveSurface: Valid only if 'SaveRestore' registry key is 1

0 - The driver will not save/restore all surfaces to/from system memory

1 - Otherwise

- FB_BASE:  For Host-based driver only!

Specify the physical address of the VGX frame buffer.

- FG_BASE:  For Host-based driver only!

Specify the physical address of the VGX register MMIO.

- PanelType: For special panel type. Certain number corresponds to certain panel type,

as listed below (all values are hex values):

0 TFT_18BIT

1 TFT_24BIT

2 TFT_36BIT

- Angle: Angle of rotation 0(0), 90(0x5A), 180(0xB4), and 270(0x10E),

only if Rotation feature is enabled.

If not specified, Angle is defaulted to 0.